FOOD PERISH PREVENTION SYSTEM USING IOT

ARDUINO CODE: 
            #include <DHT.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#define DHTPIN 2

#define ONE_WIRE_BUS 3

#define LM35PIN A0

#define RELAY_HEAT 5

#define RELAY_COOL 4

#define BTN_HOT_MODE 6

#define BTN_COLD_MODE 7

#define BTN_SANDWICH 8

#define BTN_PUFFS 9

#define BTN_MILK 12

#define BTN_DRINKS 13

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

LiquidCrystal_I2C lcd(0x27, 16, 2);

bool modeSelected = false;

bool foodSelected = false;

bool isColdMode = false;

String foodType = "";

float tempMin = 0.0;

float tempMax = 0.0;

void printLCDAndSerial(int row, const char* msg) {

lcd.setCursor(0, row);

lcd.print("                ");

lcd.setCursor(0, row);

lcd.print(msg);

Serial.println(msg);

}

void blinkAlert() {

while (true) {

lcd.clear();

lcd.setCursor(3, 0);

lcd.print("!!! ALERT !!!");

lcd.setCursor(3, 1);

lcd.print("Sensor Error");

Serial.println("!!! ALERT: Sensor error detected !!!");

digitalWrite(RELAY_COOL, HIGH);

digitalWrite(RELAY_HEAT, HIGH);

delay(700);

lcd.clear();

delay(300);

}

}

void setup() {

Serial.begin(115200);

dht.begin();

sensors.begin();

lcd.init();

lcd.backlight();

pinMode(RELAY_COOL, OUTPUT);

pinMode(RELAY_HEAT, OUTPUT);

digitalWrite(RELAY_COOL, HIGH);

digitalWrite(RELAY_HEAT, HIGH);

pinMode(BTN_HOT_MODE, INPUT_PULLUP);

pinMode(BTN_COLD_MODE, INPUT_PULLUP);

pinMode(BTN_SANDWICH, INPUT_PULLUP);

pinMode(BTN_PUFFS, INPUT_PULLUP);

pinMode(BTN_MILK, INPUT_PULLUP);

pinMode(BTN_DRINKS, INPUT_PULLUP);

lcd.clear();

printLCDAndSerial(0, "Select Mode:");

printLCDAndSerial(1, "HOT / COLD");

}

void loop() {

if (!modeSelected) {

if (digitalRead(BTN_HOT_MODE) == LOW) {

modeSelected = true;

isColdMode = false;

lcd.clear();

printLCDAndSerial(0, "HOT Mode Selected");

delay(1000);

lcd.clear();

printLCDAndSerial(0, "Select Food:");

printLCDAndSerial(1, "Puffs / Sand.");

Serial.println("Mode: HOT selected");

}

else if (digitalRead(BTN_COLD_MODE) == LOW) {

modeSelected = true;

isColdMode = true;

lcd.clear();

printLCDAndSerial(0, "COLD Mode Selected");

delay(1000);

lcd.clear();

printLCDAndSerial(0, "Select Food:");

printLCDAndSerial(1, "Milk / Drinks");

Serial.println("Mode: COLD selected");

}

return;

}

if (!foodSelected) {

if (isColdMode) {

if (digitalRead(BTN_MILK) == LOW) {

foodType = "Milk";

tempMin = 1.0;

tempMax = 4.0;

foodSelected = true;

}

else if (digitalRead(BTN_DRINKS) == LOW) {

foodType = "Drinks";

tempMin = 5.0;

tempMax = 8.0;

foodSelected = true;

}

} else {

if (digitalRead(BTN_PUFFS) == LOW) {

foodType = "Puffs";

tempMin = 60.0;

tempMax = 70.0;

foodSelected = true;

}

else if (digitalRead(BTN_SANDWICH) == LOW) {

foodType = "Sandwich";

tempMin = 50.0;

tempMax = 60.0;

foodSelected = true;

}

}

if (foodSelected) {

lcd.clear();

printLCDAndSerial(0, "Food Selected:");

printLCDAndSerial(1, foodType.c_str());

delay(1500);

lcd.clear();

}

return;

}

float lm35Temp = analogRead(LM35PIN) * (5.0 / 1023.0) * 100.0;

float dhtTemp = dht.readTemperature();

float dhtHum = dht.readHumidity();

sensors.requestTemperatures();

float ds18Temp = sensors.getTempCByIndex(0);

// Alert on sensor error

if (isnan(dhtHum) || isnan(dhtTemp) || ds18Temp == -127.0 || lm35Temp < -20 || lm35Temp > 100) {

blinkAlert();

}

bool coolOn = false, heatOn = false;

if (ds18Temp > tempMax && isColdMode) {

digitalWrite(RELAY_COOL, LOW);

digitalWrite(RELAY_HEAT, HIGH);

coolOn = true;

}

else if (ds18Temp < tempMin && !isColdMode) {

digitalWrite(RELAY_COOL, HIGH);

digitalWrite(RELAY_HEAT, LOW);

heatOn = true;

}

else {

digitalWrite(RELAY_COOL, HIGH);

digitalWrite(RELAY_HEAT, HIGH);

}

// Serial debug output

Serial.println("=== Sensor Readings ===");

Serial.print("LM35 Temp: "); Serial.println(lm35Temp);

Serial.print("DHT11 Temp: "); Serial.println(dhtTemp);

Serial.print("Humidity: "); Serial.println(dhtHum);

Serial.print("DS18B20 Temp: "); Serial.println(ds18Temp);

Serial.print("Cooling Relay: "); Serial.println(coolOn ? "ON" : "OFF");

Serial.print("Heating Relay: "); Serial.println(heatOn ? "ON" : "OFF");

Serial.print("Selected Food: "); Serial.println(foodType);

Serial.println("=======================");

// LCD Output

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(ds18Temp, 1);

lcd.print((char)223);

lcd.print("C ");

if (coolOn) lcd.print("C:ON ");

else if (heatOn) lcd.print("H:ON ");

else lcd.print("STBY  ");

char line2[17];

snprintf(line2, sizeof(line2), "H:%d%% %s", (int)dhtHum, foodType.c_str());

lcd.setCursor(0, 1);

lcd.print("                ");

lcd.setCursor(0, 1);

lcd.print(line2);

delay(5000);

}

ESP01 Wifi Module Code

#include <ESP8266WiFi.h>

#include <WiFiClient.h>

const char* ssid = "<Wifi_Name>";

const char* password = "<Wifi_Password>";

const char* host = "api.thingspeak.com";

const char* apiKey = "5WD8MGBI66TQXEHT";

WiFiClient client;

void setup() {

Serial.begin(9600); // Connect to Arduino

delay(1000);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.println("Connecting to WiFi...");

}

Serial.println("Connected to WiFi!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

}

void loop() {

if (Serial.available()) {

String data = Serial.readStringUntil('\n');

data.trim(); // remove whitespace or \r

Serial.println("Received from Arduino: " + data);

// Split the data

int idx1 = data.indexOf(',');

int idx2 = data.indexOf(',', idx1 + 1);

int idx3 = data.indexOf(',', idx2 + 1);

if (idx1 > 0 && idx2 > idx1 && idx3 > idx2) {

String val1 = data.substring(0, idx1);                    // ds18Temp

String val2 = data.substring(idx1 + 1, idx2);             // dhtTemp

String val3 = data.substring(idx2 + 1, idx3);             // lm35Temp

String val4 = data.substring(idx3 + 1);                   // dhtHum

if (client.connect(host, 80)) {

String url = "/update?api_key=" + String(apiKey) +

"&field1=" + val1 +

"&field2=" + val2 +

"&field3=" + val3 +

"&field4=" + val4;

client.print(String("GET ") + url + " HTTP/1.1\r\n" +

"Host: " + host + "\r\n" +

"Connection: close\r\n\r\n");

Serial.println("Data sent to ThingSpeak!");

Serial.println("URL: " + url);

} else {

Serial.println("Connection to ThingSpeak failed.");

}

} else {

Serial.println("Invalid data format received.");

}

delay(2000); // optional delay between uploads

}

}